CEE 311   Environmental Engineering I

Dr. Kauser Jahan

##### Laboratory # 1

##### Measurement of Water Quality Parameters

Engineering Objectives

- To learn about parameters that characterize water quality

- To measure water quality parameters using analytical equipment

Sources of Water

- Groundwater

- Surface Water

- Oceans

- Reclaimed Wastewater

Purposes for Water Treatment

- Disinfection

- Removal of Turbidity

- Removal of Color, and Tastes & Odors

- Removal of Iron & Manganese

- Hardness removal

- Protection from Toxic Organics and Inorganics

# Drinking Water Standards

## National SDWA (Safe Drinking Water Act) (Enacted in 1974)

MCL-Maximum Contaminant Levels (MCLs) enacted by USEPA with margin of safety

MCLG- Maximum Contaminant Level Goals

SWTR-Surface Water Treatment Rule

Lead and Copper Rule

## Primary

- Protection of Human Health

  - Organics: Pesticides, Solvents, Fuels

  - Inorganics: Metals.

  - Radionuclides: Beta particles, alpha particles, radon

  - Microbes: viruses, bacteria, protozoa, helminthes

### Secondary

- Related to Aesthetics

  - Taste

  - Odor

  - Clarity

  - Color

Water Quality Categories

Physical – appearance (color, taste, odor, temperature, turbidity)

Chemical – (organic, inorganic compounds)

Biological – (microorganisms)

Radiological- presence of radioactive substances

Water Quality Parameters

Turbidity - The tendency of water to scatter light at 90 degrees. Turbidity is a measure of water clarity. Caused by suspended solids (thus, turbidity is an indirect measure of suspended solids). Measured in NTUs, using a Turbidimeter. For most people, water with <= 5 NTUs looks clear. The American Water Works Association (AWWA) recommends that water to be disinfected should be <= 0.1 NTU.

Color - True color is caused by dissolved compounds in water. It can be natural or anthropocentric. Dissolved and suspended solids (together) cause apparent color. For example, brown colored water could be the result of dissolved byproducts of plant biodegradation (true color) or suspended clay particles (apparent color) or both (also apparent color). Color is measured in Platinum-Cobalt units (PCU). The AWWA recommends <= 15 Platinum Cobalt units. This is also the U.S. secondary drinking water regulation. Color can be measured using light with a wavelength of 455 nm.

Pathogens - Pathogens are disease-causing microorganisms.

pH – pH indicates the intensity of the acid or alkaline condition of a solution.    It expresses the hydrogen ion concentration.  pH is an important factor that impacts most water treatment processes such as disinfection, coagulation, water softening and corrosion control.  pH has no units.  The term may be represented by

pH = -log[H+ in moles/liter]

Alkalinity- The alkalinity of water is a measure of its capacity to neutralize acids.  Three major classes of materials cause the major portion of alkalinity in natural waters.  These are as follows: (1) hydroxide (2) carbonates and (3) bicarbonates.   Bicarbonates represent the major form of alkalinity.

Hardness - Hardness is caused by the presence of divalent cations.  Such ions are capable of reacting with soap to form precipitates with certain anions present in the water to form scale.  The main cations causing hardness are calcium, magnesium, ferrous iron and manganous ions.  The unit is expressed as mg/L as CaCO3.

Waters are normally classified in terms of the degree of hardness as follows:

| mg/L as Ca CO3 | Degree of Hardness |
| --- | --- |
| 0-75 | Soft |
| 75-150 | Moderately Hard |
| 150-300 | Hard |
| 300 up | Very Hard |

Organic Compounds- There are a number of methods for determination of organics such as the Total Organic Carbon Analyzer and the Chemical Oxygen Demand Test.  Most organic compounds show absorbance at 254 nm.

Conductivity- This is a measure of the presence of ions (cations and anions) in solution. Conductivity is used to measure the concentration of dissolved solids which have been ionized in a polar solution such as water. The unit of measurement commonly used is one millionth of a Siemen per centimeter (micro-Siemens per centimeter or µS/cm).

Nitrate- Nitrate is a primary drinking water standard.  Its presence also causes the blue-baby syndrome in infants.  Nitrate is also a nutrient for algae and can stimulate growth of algae.  The drinking water standard is 10 mg/L as the maximum contaminant level (MCL) for nitrate-nitrogen and 1 mg/L for nitrite-nitrogen for regulated public water systems.

Phosphate - Phosphate is a major source of pollution in surface waters from human activities such as irrigation and agriculture.    It is a nutrient for plants and algae and can cause eutrophication of lakes.

References:

1.  Davis, M. and Masten , S. J. (2004)  Principles of Environmental Engineering and Science, McGraw Hill.

2.  http://www.epa.gov/safewater/mcl.html#mcls

Laboratory Exercise

- Each team will obtain a sample of a certain type of water and characterize the water.

Sample Data Collection Form

Date Collected:

Team Members:

Time:

Temperature:

Weather Conditions:

| Parameter | Units | Equipment Used | Rowan Pond Water | Tap Water | USEPA Limit |
| --- | --- | --- | --- | --- | --- |
| pH |  |  |  |  |  |
| Dissolved Oxygen |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |
| Hardness |  |  |  |  |  |
| Conductivity |  |  |  |  |  |
| Turbidity |  |  |  |  |  |
| Color |  |  |  |  |  |
| Organics |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Phosphate |  |  |  |  |  |

Laboratory Report

Title Page

Letter of Transmittal

Abstract, Introduction, Objectives, Materials and Methods, Results and Discussion, Conclusions, References

Compare your results to the USEPA drinking water standards1 for tap water.  Compare the pond water to the NJ SWQS2 .  Discuss the results.

1. http://water.epa.gov/drink/contaminants/index.cfm

2. http://www.state.nj.us/dep/wms/bwqsa/swqs.htm