# CE 311 Environmental Engineering I                                                    Fall 2004

Dr. Kauser Jahan, P.E.

## MIDTERM EXAM # 1

Circle correct answer                                                               (10 points)

- The detention time  for a reactor with a flowrate of 10GPD (Gallons per day) and 20 Gallons volume is

- 2 hours

- 2 days

- 2 minutes

- The suspended solids concentration in lbs/day in a water treatment plant with a flowrate of 10MGD and a suspended solids concentration of 60 mg/L is

- 5,004 lbs/day

- 30,024 lbs/day

- 50,000 lbs/day

- The flowrate of a river is 15 ft3/min.  The flowrate in gallons/day is

- 1,600 gallons/day

- 161,568 gallons/day

- 2,887 gallons/day

- If the calculated diameter for a reactor is 15.58 feet,  the design diameter in feet is

- 15.58 feet

- 15 feet

- 16 feet

e)  The normality of a 2000  mg/L sulfuric acid solution is

(i)  1N

(ii)  0.041N

(iii)  5N

2.                                          				                        (10 points)

(a)  A 10 MGD wastewater treatment plant has a suspended solids concentration of 80 mg/L.  If the plant discharges into the Delaware river (Upstream Flow 3 MGD, Upstream Suspended Solids 5 mg/L), what is the suspended solids concentration in mg/L downstream after the plant discharge?

Delaware River

Wastewater Plant

(b)  If the New Jersey regulatory agency allows a suspended solids concentration of 30 mg/L in the downstream reach of the river, what concentration of suspended solids in mg/L can the wastewater treatment plant discharge?

(3)                                                                                                             15 points

A Gook manufacturing facility obtained the following plots for determining the reaction rate for gook removal in their wastewater.

- What is the order of the reaction?  ___________________

- (b) What is the value of the reaction rate?  ____________   Units _________

- What was the initial gook concentration in mg/L at the start of the reaction?

- If the reaction rate for gook was 0.057 day-1, what is the value of its half-life in days?

4.                                                                                                       (10 points)

A 12.0 MGD water treatment plant has an influent BOD5 of 250 mg/L.  A circular reactor with the dimensions shown has been designed for BOD5 removal.

75 feet

50 feet

(a) What is the volume of the reactor in gallons?

- What is the detention time of the reactor in hours?

- The plant needs to reduce the BOD5 by 90%.  What is the effluent BOD5 concentration in mg/L?

5.                                                                                                           (10 points)

A 10 MGD wastewater treatment plant has an influent organic concentration of 200 mg/L.  The regulatory limit for organics in the effluent is 25 mg/L.  The plant currently has a reactor that can remove only 70% of the organics.  The plant is thinking of adding a new reactor in series to the existing one.

OLD

70% Organics removal

NEW

Influent

Regulatory limit 25 mg/L for organics

(a) What % organics removal must occur in this new reactor to meet the regulatory limit?

- If both reactors have the same volume, will they have the same detention time? Justify your answer.

- Two diffusers were tested for gas transfer efficiency.  The plot for obtaining the values of the gas transfer coefficient is shown below:

- What is the value of the gas transfer coefficient in hour-1 for the two diffusers?

(b)  If your plant requires a diffuser with a gas transfer rate of 15.5 hr-1, which diffuser would you choose?  Why?

- An industrial wastewater treatment process uses activated carbon to remove color from the water.  The color is reduced as a first order reaction in a batch reactor.  If the reaction rate is 0.35 day-1 how long will it take to remove 90% of the color?

8.    A CMFR is operating at steady state at a flowrate of 4 L/min and at an inflow gloop concentration of 400 mg/L.  The volume is 60 L.  The reaction is first order and the gloop concentration in the reactor is 100 mg/L.

- What is the reaction rate in min-1?

- What is the detention time in hours?

- What is the % removal of gloop?

- If an identical reactor was added to the existing one, what would the new gloop concentration be in the effluent?  What is the overall % removal?

10.  If the Garden River water has an ultimate BOD of 385 mg/L at 200C, what is the BOD5 in mg/L?  Assume k=0.23 day-1 at 20oC.  If the value of theta is 1.024, what is the value of the BOD5 at 10oC?

11.  Chemical Analyses of the river water quality indicates that the water has 150 mg/L of the compound indicated below.   Determine in mg/L the carbonaceous and nitrogenous demand exerted by the compound.

H

Structure            I

of Glycine	H-C – COOH

I

NH2

BONUS

The wastewater from a textile manufacturing plant will be discharged to the Garden City Plant.  The characteristics of the textile wastewater are 1,500 mg/L BOD5, 2,000 mg/L TSS, 30 mg/L nitrogen, and no phosphorus.  If the required BOD/N/P weight ratio is 100/5/1, what is the minimum quantity of domestic wastewater required per 1000 gallons of textile wastewater to provide adequate nutrient for biological growth? [Assume typical values for the wastewater  BOD5 = 220 mg/L, N=40 mg/L and P=8 mg/L]