In-Class Exercise on Gas Transfer

1.  Aeration kinetics can be expressed as follows:

ln(C*-Ct) = ln(C*-Co) – KLa t  where

C* = saturation concentration of oxygen in water (mass/volume)

Ct = oxygen concentration in water at ant time t (mass/volume)

KLa = gas transfer coefficient (time-1)

t= time

Your team has plotted data collected from a gas transfer experiment as shown below and has obtained the equation of the best-fit straight line.

What is the value of KLa in min-1 that can be obtained from this plot?  Write appropriate UNITS!

Value of KLa  = slope of line = 0.2331 sec-1  =   0.2331 * 60 sec  = 13.98 min-1

Sec          min

- Two diffusers are to be tested for their oxygen transfer capability.  Tests were conducted at 20oC with the following results:

|  | Dissolved Oxygen mg/L |  |
| --- | --- | --- |
| Time (minute) | Air Max | Wonder |
| 0 | 2.0 | 3.5 |
| 1 | 4.0 | 4.8 |
| 2 | 4.8 | 6.0 |
| 3 | 5.7 | 6.7 |

Cs  = 9.2 mg/L at 20oC

| Time (minute) | Air Max | Cs-C | ln(Cs-C) | Wonder | Cs-C | ln(Cs-C) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2 | 7.2 | 1.97408 | 3.5 | 5.7 | 1.740466 |  |
| 1 | 4 | 5.2 | 1.64866 | 4.8 | 4.4 | 1.481605 |  |
| 2 | 4.8 | 4.4 | 1.4816 | 6 | 3.2 | 1.163151 |  |
| 3 | 5.7 | 3.5 | 1.25276 | 6.7 | 2.5 | 0.916291 |  |

Slope of line is the value of Kla.   Wonder Diffuser  has a higher KLa.