# CEE 311  Environmental Engineering  I

Dr. Kauser Jahan, P.E.

In Class Exercise:

Oxygen Demand and  DO Sag Curve

- Determine the theoretical oxygen demand for 150 mg/L of glucose.

C6H12O6  + 6 O2 = 6CO2  +  6H2O

MW of glucose   = 180 g/mole

Therefore THOD  = 6*32 g/mole O2    * 150 mg/L glucose   =     160 mg/L

180 g/mole glucose

## 2.  The BOD5 of a sewage sample is 200 mg/L.  What is the ultimate BODu at 20oC if k20 = 0.16 day-1?  What is the BOD5 and BODu  of the sample at 30oC if k30 is 0.25

## day-1?

Lt = Lo (1-e-kt)   Given  Lt = 200  Lo=?   k20 = 0.16 day -1   t= 5 days

200 = Lo(1-e-.16*5)  or  Lo = 363.19 mg/L  = BODu

Note BODu is not a function of temperature.  Therefore same BODu will be used at 30oC

BOD5  =Lt = 363.19 (1 – e-.25*5)  = 259.13 mg/L

3.

## Dissolved Oxygen

1.  What is the value of the DO at saturation level if the temperature of the stream is 15oC?

LOOK UP VALUE IN BOOK  10.15 mg/L

2.  What is the value of the initial DO deficit in mg/L? 10.15 -8.0 =2.15 mg/L

3.  What is the value of the critical DO?   4.0 mg/L

- What is the value of the maximum deficit of DO in the stream in mg/L?

10.15-4.0=  6.15 mg/L   (4.0 is the DO at the lowest point of the curve)

- What is the distance in miles where the maximum deficit occurs?  100 km

- If the velocity of the stream is 0.065 km/sec at what is the value of critical time in days?

Critical Time = 100 km  * sec *min  * hr    *     day          =  0.0178 days

0.065 km  * 60 sec  * 60 min  * 24 hrs

- What causes oxygen depletion in a stream when an organic wastewater is discharged?  Draw a qualitative DO sag curve.

- Name three factors that impact the solubility of oxygen in water?  Pressure, temperature, salinity

- A waste stream has a dissolved oxygen concentration of 1.5 mg/L, a flow of 0.5 m3/sec, a temperature of 26 oC and an ultimate BOD of 48 mg/L.  The stream water is running at 2.2 m3/sec at a saturated DO, a temperature of 12oC and an ultimate BOD of 1.6 mg/L.  Calculate the dissolved oxygen concentration 48.3 km downsteam.

Given k1 0.2 day-1  and k2  = 0.4 day-1

Find mixing temperature, DO and BOD

Find corresponding DOsat  for mixing temp. to calculate mixing DO.

Calculate DO deficit using equation

Mixing Temperature  = 0.5 m3/sec* 26 oC  + 2.2 m3/sec*12oC  =  14.6oC  = 15 oC

(0.5+2.2) m3/sec

Mixing BODu         = 0.5 m3/sec* 48mg/L  + 2.2 m3/sec*1.6 mg/L  =  10.19 mg/L

(0.5+2.2) m3/sec

Look up DOsat at 12oC from book  = 10.83mg/L

Mixing DO  = 0.5 m3/sec* 1.5 mg/L  + 2.2 m3/sec*10.83mg/L  =  9.10 mg/L

(0.5+2.2) m3/sec

At a temperature of 15oC  DOsat = 10.15 mg/L

Therefore Initial deficit = 10.15-9.1 = 1.05 mg/L

Time t = 48 m/ 0.37 m/s   = 129.73 sec  = 0.0015 days

Using equation 8-33  Deficit at48 m  =  1.053 mg/L

Therefore DO at 48 km = 10.15-1.053 = 9.09 mg/L

Use eqn 8-35 and 8-37 for critical time and critical deficit

tc = 2.92 days     Dc = 2.84 mg/L

Therefore DO at critical point = 10.15-2.84 = 7.31mg/L > 4mg/L required for aquatic life