Study Guide on Water Softening

- Name two impacts of using hard water.  Soap Consumption, Clogging of pipes

- What causes hard water?  Polyvalent cations, Common Ca and Mg

- Name two processes that can be used to remove hardness.

Chemical Precipitation, Ion Exchange

- Draw a bar diagram (all constituents expressed as mg/L of CaCO3) and determine the lime and soda-ash requirements in mg/L of each constituent for a water that has the following constituents.  Report all forms of hardness.

| Component | mg/L | Equivalent Weight | mg/L as CaCO3 |
| --- | --- | --- | --- |
| CO2 | 8.8 | 22 | 20 |
| Ca+2 | 70 | 20 | 175 |
| Mg+2 | 9.7 | 12.2 | 39.75 |
| Na+ | 6.9 | 23 | 15 |
| Alk (HCO3-) | 115 | 61 | 94.26 |
| SO42- | 96 | 48 | 100 |
| Cl- | 25.19 | 35.5 | 35.49 |

Eq. Wt of Lime 28      Eq. Wt. of  Soda Ash  53

Calculate concentrations in mg/L of CaCO3 for each constituent

CO2  = 8.8 * 50/22  = 20 mg/L

Ca+2 = 70*50/20  =  175

0                                                      175                   214.75        229.75

Ca+2

Mg+2

Na+

CO2

20

HCO3-

SO4--

Cl-

0                                          94.26                         194.26        229.16

Total Hardness  = (Ca + Mg) = 214.75 mg/L as CaCO3

Carbonate Hardness = 94.26 mg/L as CaCO3

Non Carbonate Hardness = (214.75-94.26)  =  120.49 mg/L as CaCO3

Names of constituents (and concentration) that cause hardness

Lime			Soda Ash

CaHCO3        94.26                          1				x

Ca SO4          80.74			x				1

Mg SO4         19.26			1				1

MgCl2              20.49			1				1

CO2              20.00			1

CO2 does not cause hardness but consumes lime!!!

Lime Requirements

(94.26+19.26+20.49+20)  =   154.01  mg/L as  CaCO3  = 154.01*28/50  = 86.25 mg/L as lime

Soda Ash  (80.74+19.26+20.49) = 120.49 mg/L as CaCO3  = 120.49*53/50 = 127.72 mg/L as soda ash

- If a 10 MGD water treatment plant requires 35 mg/l of lime for water softening what is the requirement in lbs/day?

10 MGD * 35 mg/L * 8.34  = 2919 lbs/day