Bandwidth Distribution Scheme for Dynamic, Scalable, and Fair Allocation of Bandwidth

- Vasil Hnatyshin1 and Adarshpal S. Sethi2

- 1Department of Computer Science

- Rowan University

201 Mullica Hill Rd.

- Glassboro, NJ 08028

- hnatyshin@rowan.edu

- 2Department of Computer and

- Information Sciences,

- University of Delaware,

- Newark, DE 19716

- sethi@cis.udel.edu

- ABSTRACT

Despite numerous efforts, the problem of providing per-flow Quality of Service in a scalable manner still remains an active area of research. This paper introduces a novel approach, called the Bandwidth Distribution Scheme (BDS), for providing per-flow bandwidth guarantees in a scalable manner. In a version of BDS named the X-BDS, he edge routers keep per-flow information, while the core routers maintain the aggregate flow requirements. The X-BDS approach employs a distributed message exchange protocol for providing network feedback and for distributing aggregate flow requirements among the nodes in the network. Based on the obtained feedback the edge routers employ the X-BDS resource management unit to dynamically distribute available bandwidth among individual flows. The X-BDS admission control and resource management units are responsible for fair resource allocation that supports minimum bandwidth guarantees of individual flows. This paper evaluates the Bandwidth Distribution Scheme through simulation and shows that the X-BDS is capable of supporting scalable per-flow bandwidth guarantees.

Keywords: Quality of service, bandwidth distribution, network feedback

- Introduction

To solve the problem of providing scalable per-flow Quality of Service, a number of service differentiation models have been proposed. The Integrated and Differentiated Service models are among the most prominent approaches to providing Quality of Service in the Internet. The Integrated Services (IntServ) model [3] requires each router in the network to reserve and manage resources for the flows that travel through it. In large networks, millions of flows may simultaneously travel through the same core routers. In such cases, managing resource reservations on a per-flow basis may cause enormous processing and storage overheads in the core routers. As a result, the IntServ is considered to be not scalable to large networks and thus is not widely deployed in the Internet. The Differentiated Services (DiffServ) model [2] attempts to solve the scalability problem of the IntServ approach by combining flows that have similar quality of service requirements into the traffic aggregates or classes. The DiffServ core routers process incoming traffic based on the class the packets belong to and thus maintain and manage resource reservations only on a per-class/per-aggregate basis. Although the Differentiated Services approach provides a scalable solution to the QoS problem, it supports only coarse per-aggregate guarantees which in certain cases may not be adequate.

This paper introduces an alternative approach for providing scalable per-flow bandwidth guarantees, called the Bandwidth Distribution Scheme (BDS). In this approach, the core routers do not maintain per-flow information (e.g. bandwidth requirements of individual flows); instead core routers keep aggregate flow requirements. The amount of information kept in the network core is proportional not to the number of flows but to the number of edge routers, which we believe does not raise scalability concerns. The edge nodes maintain per-flow information and fairly allocate network resources (e.g. bandwidth) among individual flows according to the flow requirements and resource availability. The X-BDS relies on the idea of pushing per-flow information to the network edges while keeping traffic aggregate information in the network core. This idea is not new and has been examined before, for instance in [2, 5, 6, 17, 22]. However, the primary contribution of this paper is a novel approach to aggregating flow requirements and a new distributed network feedback protocol that allows the edge nodes to discover network changes and compute fair per-flow bandwidth allocation that satisfies minimum bandwidth guarantees of individual flows.

We have designed and studied two different variations of BDS, named Estimation-Based BDS (S-BDS) and Exact Requested Bandwidth Range BDS (X-BDS). In X-BDS, the edge routers obtain the exact values of the aggregated flow requirements as contrasted with S-BDS, where the edge routers estimate the aggregate flow requirements. This paper describes the X-BDS approach and presents its performance evaluation via simulation (the S-BDS approach has been described in our earlier publications [8, 11, 12]. The X-BDS variation improves on the performance of the S-BDS approach in several categories. In particular, the X-BDS provides a more accurate computation of the flow fair shares, converges to the optimal rates faster, and extends the basic BDS architecture by including the admission control unit.

The X-BDS architecture provides a scalable solution to the problems of fair per-flow bandwidth distribution and congestion control. This architecture (Figure 1) consists of three components: a set of specifications and definitions, the resource management mechanism, and the Requested Bandwidth Range (RBR) Distribution and Feedback (RDF) protocol. The X-BDS specifications and definitions consist of the network architecture which defines the working environment of the X-BDS and makes the proposed solutions scalable to large networks, definition of the flow requirements which outlines the format for user expectations for traffic treatment, and definitions of fairness which specify what it means for the resource distribution to be fair. The X-BDS resource management mechanism consists of per-flow admission control that denies access into the network for those flows that violate existing per-flow guarantees and per-flow resource allocation that dynamically allocates available bandwidth to individual flows. The Requested Bandwidth Range (RBR) Distribution and Feedback protocol (RDF) is the glue that holds the X-BDS approach together. The RDF protocol is a distributed explicit message exchange protocol that provides feedback about the changes of network characteristics. More specifically, the RDF protocol distributes aggregate flow requirements among the routers in the network and generates explicit congestion notifications when needed.

Scalability

Fair Per-flow

Bandwidth Distribution

Congestion Control

BDS Goals

RBR Distribution and Feedback Protocol

X-BDS Architecture

Flow Requirements

Definitions of Fairness

Network Architecture

Specifications and Definitions

Admission Control

Resource Allocation

Resource Management Mechanism

Figure 1. The X-BDS architecture

The rest of the paper is organized as follows. Section 2 introduces the X-BDS specifications and definitions. The resource management mechanism and the protocol for distribution of the aggregate flow requirements are described in Sections 3 and 4, respectively. In Section 5 we introduce implementation details, while Section 6 presents performance evaluation of the X-BDS approach. Finally, Section 7 provides related work overview and discussion while Section 8 presents the conclusions.

- the X-BDS Specifications and Defintions

## Network Architecture

The X-BDS approach uses a network architecture similar to that of the Differentiated Services model [2], where the Internet consists of a large number of routers grouped into independent network domains as shown in Figure 2. A cluster of interconnected routers that are governed by the same administrator is called a network domain. Each network domain contains two types of nodes: the edge or boundary routers and the core routers. Traffic enters a network domain through the edge nodes called ingress routers, travels through the core routers to reach the network boundary, and exits the domain through the edge nodes called egress routers. This paper examines the Exact-RBR Bandwidth Distribution Scheme within the confines of a single network domain. We plan to address the issue of inter-domain traffic and deployment of the X-BDS across multiple domains in future work.

Aggregate

Per-Edge Node

Information

Per-Flow

Information

Network Domain

Ingress A

Core 1

Ingress B

Core 2

Core 3

The Internet

Network

Domain

Network

Domain

Network

Domain

Figure 2. The X-BDS Network Architecture

In a network domain, the X-BDS core routers do not perform per-flow management. Instead each core router maintains information about the edge routers that send traffic through its links. We believe that the number of edge routers that send traffic through a particular router is relatively small and will not cause scalability concerns. In addition, the X-BDS core routers do not require any additional processing of data packets. The primary responsibility of the core routers is to maintain and distribute the aggregate flow information, which requires a small amount of additional processing associated with managing the RDF protocol’s packets. In the X-BDS network domain, the edge routers are responsible for maintaining per-flow information and regulating data traffic that enters the network. Each edge router controls the flow of data traffic by dynamically adjusting bandwidth allocation of individual flows, which requires per-flow information stored at that edge router and aggregate information provided by the core routers.

It is reasonable to assume that the number of active flows that enter and exit the network domain through a particular edge router is fairly small. Thus, managing per-flow information at the network boundaries allows such network architecture to scale well to large networks [2]. Furthermore, such network architecture does not require the X-BDS approach to be set-up everywhere in the Internet at once. Instead, each network domain can choose to support the Bandwidth Distribution Scheme at its own discretion, and thus facilitates incremental deployment. If a network domain decides to support the X-BDS, then a certain amount of resources (e.g. bandwidth) are allocated for the X-BDS traffic. These resources are fairly distributed among the X-BDS flows only, thus isolating the X-BDS traffic from the rest of the non-BDS flows traveling through this domain.

This paper defines a "flow" to be a sequence of packets that travel from a given source host to a given destination host. We only consider the flows that receive the X-BDS treatment and which are therefore subject to the X-BDS resource allocation. Similarly, the terms “resources”, “capacity”, “load,” or “bandwidth” mean the resources, bandwidth, etc. explicitly allocated by the network administrator for the X-BDS traffic. This definition of a flow, while different from the more conventional definition as a sequence of packets between individual source-destination applications (e.g., TCP or UDP streams), was chosen to simplify the presentation of the X-BDS scheme. The X-BDS architecture, as presented here, can be easily extended to apply to the conventional definition of a flow.

## The X-BDS Flow Requirements and Definitions of Fairness

In this paper we assume that both the minimum and the maximum transmission rates of a flow are known ahead of time. Thus, the flow requirements are defined in the form of a range called the Requested Bandwidth Range (RBR). The RBR of flow f, RBRf, consists of two values: a minimum rate, bf, below which the flow cannot operate normally, and the maximum rate,Bf, that the flow can utilize.

RBRf=[bf,Bf]						(1)

Link k is a bottleneck of flow f if link k limits transmission rate of flow f. Consider a core router’s interface k and a set of flows, Fk, that travel through it. The set Fk can be divided into two disjoint subsets: the subset, FBk, of flows that have link k as their bottleneck and the subset, FNBk, that contain all the other flows. These subsets are called bottleneck flows and non-bottleneck flows, respectively.

Fk=FBk∪FNBk					(2)

The aggregate bottleneck RBR and the aggregate RBR on interface k are defined as follows:

bBk=f∈FBkbf		BBk=f∈FBkBf			(3)

bk=f∈Fkbf		Bk=f∈FkBf			(4)

The aggregate bottleneck RBR is the sum of the RBRs of the bottleneck flows on link k, while the aggregate RBR is the sum of the RBRs of all the flows that travel through link k. The total allocated rate of the non-bottleneck flows is called the non-bottleneck rate and is denoted as RNBk. The amount of bandwidth left for distribution among the bottleneck flows is the difference between the capacity of link k and the non-bottleneck rate. This value is called the bottleneck capacity, CBk.

CBk=Ck−f∈FNBkRf=Ck−RNBk			(5)

Generally, the goal of fairness is to provide an amount of resources to a customer that is proportional to the price paid by the customer [1]. In the X-BDS approach, if customers are charged based on their desired RBR’s, the fairness goal is realized by allocating each bottleneck flow f on link k an amount of excess bandwidth proportional to the flow’s weight wf, which is a function of the flow’s RBR and thus the price. The excess bandwidth is the resources left after each bottleneck flow is allocated its minimum rate. The fair share of flow fthrough bottleneck link kis then:

FSfk=bf+(CBk−bBk)wfj∈FBkwj	(6)

It is interesting to note that, although we crafted the fairness definition (6) independently to suit the specific requirements of X-BDS, it is equivalent to the definition of general weighted fair allocation in ATM networks [25]. Based on the fairness definition (6), we introduce the following fairness criteria for bandwidth distribution. The simple and most intuitive way is to distribute bandwidth proportionally to the flow’s minimum requested rate (MRR) [1, 25]. We call this the proportional fairness criterion. This fairness criterion is achieved by assigning wf=bf.

FSfk=bf+(CBk−bBk)bfbBk=CBkbfbBk		(7)

This definition of proportional MRR fairness criterion (7) makes it identical to the general weighted fairness proportional to MCR criterion [25], where each flow is allocated its minimum requested rate bf plus a share of the leftover bandwidth proportional to bf.

In the second fairness criterion, the leftover bandwidth is distributed proportionally to the difference between the flow’s maximum and minimum requested rates, i.e., the amount of bandwidth a flow needs to be completely utilized. We assume that a flow is completely utilized when it sends traffic at its maximum requested rate, Bf. This fairness criterion is called maximizing utility fairness and is achieved by assigning wf=Bf−bf.

FSfk=bf+CBk−bBkBf−bfBBk−bBk	(8)

The allocated rate of a flow is limited by its RBR and thus may be equal to its maximum requested rate but smaller than its fair share on the bottleneck link.

- The X-BDS RESOURCE MANAGEMENT MECHANSIMS

## Admission Control

Based on definition (1) the X-BDS network guarantees that each flow would receive at least its minimum requested rate, bf, while the leftover resources in the network are fairly distributed among participating flows. To achieve these guarantees, the network allocates to each flow an amount of bandwidth not smaller than the flow’s minimum requested rate, and denies network access to those flows whose minimum rate guarantees cannot be met.

The purpose of admission control is to determine if a new flow can be admitted into the network at its minimum rate without violating existing QoS guarantees of other flows. The problem of admission control has been extensively examined in the literature [4, 7, 16]. Traditionally, there are two types of admission control: parameter-based and measurement-based. In parameter-based admission control, the decision to admit a new flow is derived from the parameters of the flow specification. Usually, this type of admission control relies on worst-case bounds and results in low network utilization, although it does guarantee supported quality of service. Measurement-based admission control relies on measurements of the existing traffic characteristics to make the control decision. Measurement-based admission control supports higher network utilization but it may occasionally cause the quality of service levels to drop below user expectations because of its inability to accurately predict future traffic behavior.

In the X-BDS approach, to determine if a new flow can be admitted into the network, the edge node examines the current resource allocation on the flow's path. Since the network guarantees that each flow will receive at least its minimum requested rate, the edge router verifies that the sum of the minimum requested rates of all the flows that follow a particular path, including a new flow, is smaller than the capacity of the bottleneck link on that path. Link k is a bottleneck link for flow f traveling on path P if k limits transmission rate of f on P.

Formally, we define the X-BDS admission control as follows. Consider a network that consists of a set of L unidirectional links, where link k∈L has capacity Ck. This network is shared by the set of flows, F, where flow f∈F has the RBR of [bf,Bf]. At any time, the flow enters the network at a rate Rf, called the allocated rate, which lies between bf and Bf. Let Lf⊆L denote the set of links traversed by flow f on its way to the destination. Also let Fk⊆Fdenote the set of flows that traverse link k. Then a new flow φ with the RBR of [bφ,Bφ] is accepted in the network if and only if:

bφ+f∈Fkbf≤Ck  	 ∀k∈Lφ				(9)

Thus, new flow φ is admitted into the network only if the sum of the minimum requested rates of the active flows, including the new flow, is not larger than the capacity of each link on φ's path to its destination. Equation (9) is often called the admission control test. The X-BDS approach employs a variation of the measurement-based admission control. However, unlike traditional measurement-based approaches, the X-BDS admission control does not violate bandwidth requirements of individual flows because the RDF protocol supplies accurate values of link capacities and the aggregate RBRs on the path.

## The X-BDS Resource Allocation Mechanism

To distribute bandwidth according to fairness criteria (7) an (8), the resource management mechanism requires the knowledge of such link characteristics as the aggregate bottleneck RBR and the bottleneck capacity. However, these characteristics are not readily available in the network. Instead the core routers keep track of the capacity, the arrival rate, and the aggregate RBR for each of their outgoing links. As a result, the edge nodes use the aggregate RBR and link capacity, and not the aggregate bottleneck RBR and the bottleneck capacity, to compute the fair shares of individual flows. To achieve fair resource distribution (6), the X-BDS resource management mechanism consists of two parts: the primary and leftover bandwidth distribution.

When a new flow activates or a congestion notification arrives, the edge node employs the primary bandwidth distribution and computes the fair share of flow f on its bottleneck link k using either the proportional or the maximizing utility fairness criterion as shown below.

FSfk=bf+(Ck−bk)bfbk=Ckbfbk			(10)

FSfk=bf+Ck−bkBf−bfBk−bk			(11)

However, the flows that do not have link kas their bottleneck do not adjust their allocated rates. Link k is a bottleneck link for flow f traveling on path P if k limits transmission rate of f on P or if fair share of flow f on link k is the smallest on path P. As shown in Figure 3, the edge routers maintain complete path information for each flow which is sufficient to identify the bottleneck link [8].

Clearly, if the edge routers employ only the primary bandwidth distribution as defined by equations (10) and (11), then link k may often become underutilized because the non-bottleneck flows will not adjust their allocated rates and will traverse link kat rates below their fair shares on link k. A “water-filling” technique of the max-min fairness [18, 19, 24] provides an adequate solution to this problem. The idea of the “water-filling” is to gradually increase allocated rates of individual flows until the bottleneck link is not fully utilized. The X-BDS approach periodically probes the path to discover excess bandwidth, which enables the edge routers to implement a variation of the “water-filling” technique.

The periodic path probing delivers information about availability of resources on the path and enables distribution of leftover bandwidth. If the periodic path probe discovers excess bandwidth EBkon the path (e.g. on link k), then using proportional and maximizing utility definitions of fairness, the flows that travel on that path may increase their fair shares as follows:

FSfk=FSfk+EBkbfbk			(12)

FSfk=FSfk+EBkBf−bfBk−bk			(13)

Thus, in the presence of leftover bandwidth, the edge routers increase allocated rates of individual flows until available bandwidth on the path is consumed completely. It was shown in [8] that combination of the primary and leftover bandwidth distribution as defined by equations (10) – (13) approximate fair resource distribution as defined by equations (7) and (8).

- The RDF Protocol

The RBR Distribution and Feedback (RDF) protocol is a distributed explicit message exchange protocol that governs the information sharing between the routers in the X-BDS network. The RDF protocol operates as the “glue” that holds the X-BDS architecture together by distributing information which enables the edge routers to perform admission control and fair per-flow bandwidth distribution. The protocol consists of three distinct and independent phases or parts: the path probing phase, the update phase, and the notification phase. The path probing phase discovers characteristics of a particular path. The edge routers initiate the RBR update phase to notify the core routers about the changes to the aggregate information due to flow activation or termination. In the event of congestion the core routers initiate the notification phase which distributes aggregate flow information among the corresponding edge nodes and subsequently triggers re-distribution of available bandwidth among the flows that contribute to congestion.

## The Path Probing Phase

The edge routers initiate the path probing phase to discover characteristics of the not-yet-discovered path. Throughout the duration of the path probing phase, the edge routers periodically send the probe messages to collect aggregate information maintained at each visited core router. The duration of the path probing phase is equal to the time the corresponding route remains active. A route is considered to be active if there is at least one flow that travels on this route to reach its destination. Thus, the edge routers initiate the path probing phase when a new flow that travels over the not-yet-discovered path activates. The edge routers terminate the path probing of a particular route when there is no more traffic traveling on that route.

The edge routers maintain the following tables to store information collected during the path probing phase: the SLA table which contains per-flow information, the Path table that contains information about each active path and a list of flows that travel on that path, and the Link table that contains information about the links that are part of the active paths. Figure 3 shows a simplified view of the data structures maintained at the edge nodes.

Flow List

…

Link Table

…

Data

Data

IP Address

Data

IP Address

IP Address

SLA Table

…

…

…

Source

Source

Source

Destination

Destination

Destination

Data

Data

Data

Path Table

Path

…

…

…

Complete Path

Figure 3. Data structures maintained at the X-BDS edge nodes

## The Update Phase

The purpose of the update phase is to notify the core routers about the aggregate information changes. Generally, the edge routers initiate the update phase when a new flow is being admitted into the network. The edge routers terminate the update phase after corresponding flows deactivate. During the update phase the edge routers send two control packets: one upon activation and the other upon termination. These packets are called the CNG messages and they carry the change of flow(s) requirements on the corresponding path. An optimization of the update phase allows a single CNG message to carry the requirements change for multiple flows that travel on the same path. This optimization is described in Section 5.

…

IP Address

Data

IP Address

Data

IP Address

Data

Interfaces Table

Link Capacity

Aggregate Information (2 values of type double)

Arrival Rate on the link

“Soft state” for Edge Nodes that send traffic via the link

…

Figure 4. Data structures maintained at the X-BDS core nodes

Upon the CNG message arrival, the core routers accumulate flow(s) requirements within the aggregate information of a particular link. To store the aggregate information supplied by the CNG messages the core routers maintain the Interfaces table as shown in Figure 4.

## The Notification Phase

In the event of congestion the core routers initiate the notification phase. The primary purpose of the notification phase is to eliminate network congestion by signaling the edge routers to throttle the flows that contribute to congestion. The notification phase terminates when all the edge routers that contribute to congestion receive notification messages and adjust the bandwidth allocation of their flows accordingly.

When the core router discovers congestion on one of its interfaces it consults the Interfaces table to identify the edge routers to be notified about congestion. The core router sends a control packet called the CN message to each of the identified edge routers. The CN message carries the identity and the aggregate information for the congested link. Alternatively, the CN message could be aggregated on the common part of the path to the edge routers [8, 10].

- BDS Implementation

## BDS implementation at the edge and core routers

To implement the X-BDS approach the flow requirements stored at the edge routers should include the flow RBR values. During the update phase the CNG messages carry and distribute the flow(s) RBR values among the core routers. It should be noted that upon flow termination the flow(s) RBR values carried in the CNG message are negative.

Upon the CNG message arrival the core routers update the Interfaces table and corresponding aggregate flow requirements. Specifically, the core routers add the RBR values provided by the CNG message to the aggregate RBR (e.g. the aggregate flow requirements) of the corresponding interface. In addition, the core routers maintain a “soft state” for each edge node that sends traffic on the corresponding interface. Thus, the arriving CNG message also refreshes the “soft state” timer associated with the edge router that generated this message. The edge node information is removed from the Interfaces table after a “soft-state” timer expires. Since the aggregate RBR consists of only two values of type double, the “soft state” of the edge routers is the per-edge node information maintained in each core router. We believe that storing such per-edge node information in the network core does not cause scalability concerns.

## Optimizations of the RDF protocol

During the path probing and notification phases, the aggregate RBR values of the core routers are distributed among the corresponding edge routers. That is why we call the RDF protocol the RBR Distribution and Feedback protocol: the edge nodes distribute the flow(s) RBR among the core routers during the update phase, while during the path probing and notification phases the core routers provide feedback to the edge routers by distributing the aggregate RBR values.

The RDF protocol provides a distributed solution to the problem of maintaining and sharing aggregate flow information among the routers in the network. More specifically, there is no central coordination node responsible for aggregating and distributing flow information. Each core router aggregates information only of those flows that travel through its interfaces; as a result, each core router maintains and shares information about different flow aggregates.  Furthermore, each edge router initiates its corresponding phase of RDF protocol independently of other routers in the network based on its specific internal events.

Although all the RDF protocol phases execute independently of each other, it is the case that execution of one phase triggers another. For example, the edge routers always probe the network before the update phase if characteristics of the path are unknown. Likewise, if flow activation causes congestion, then the notification phase is preceded by the update phase. Based on this observation, we implemented the following optimization of the RDF protocol. Upon activation of the new flow and before the update phase, the edge router retrieves information about the new flow’s path to the destination, which includes capacity and the aggregate RBR of each link on the corresponding path. This information allows the edge router to determine if admission of a new flow into the network will cause congestion. In the case of upcoming congestion, the edge routers notify corresponding core routers via CNG message of the update phase but delay admission of the new flow into the network. Subsequently, the core routers initiate notification phase and force the edge routers to throttle their flows before the new flow is admitted into the network. As a result, the new flow enters the network at about the same time when existing flows that share the bottleneck links with the new flow adjust their transmission rates. In effect, such optimization eliminates congestion occurrence upon flow activation [8, 10].

The X-BDS approach is ideal for long heavyweight flows such as voice and video traffic or large data transfers. In the presence of a large number of short bursty flows, the update phase of the RDF protocol may cause scalability concerns because it initiates upon each flow activation or termination. To solve this problem, the edge nodes may combine requests of multiple short flows into a single update phase. Thus, a single CNG message may carry combined information (e.g. aggregate RBR) about multiple activations and terminations of the flows that travel on the same path. This optimization, called the message aggregation technique [9], requires the edge routers to provide another level of aggregation for the short and bursty traffic. However, it has no effect on the processing in the network core and operation of the RDF protocol. In the presence of short-lived and bursty traffic the message aggregation technique improves scalability of the X-BDS approach by reducing the control load and processing during the update phase [9].

[0.5, 1.3] Mbps

T = [60 s, 330 s]

Destination 2

[0.4, 1.2] Mbps

T = [110 s, 400 s]

Destination 4

[0.2, 1.0] Mbps

T = [210 s, 400 s]

Destination 3

[0.8, 2.0] Mbps

T = [160 s, 260 s]

Destination 1

Edge 4

Edge 6

Edge 5

Edge 1

Edge 2

Edge 3

Destination 1

Core 2

Core 5

Core 3

Source 1

Source 2

Source 3

Source 4

Destination 3

Destination 4

Destination 2

Figure 5. Simulation topology

- Evaluation of the X-BDS approach

## Simulation Set-up

We study the performance of the X-BDS approach using the OPNET Network Simulator [19]. Figure 5 and Table 1 present the network topology and flow configuration used in our study. To simplify the notation, the flow that originates from source i is denoted as flow Fi. For example, the flow that originates from Source 1 is denoted as F1 and the flow of Source 2 as F2. Additionally, links Core 2 – Core 5 and Core 5 – Core 3 are denoted as links C2-C5 and C5-C3, respectively. Each link in the network is provisioned with 1600 Kbps of bandwidth (e.g., link capacity is 2000 Kbps, 80% of capacity is allocated for the X-BDS traffic). Duration of the simulation is 400 seconds.

Table 1. Flow Configuration

| Flow Name | Activation Time (seconds) | Termination Time (seconds) | Flow RBR (Kbps) | Source | Destination |
| --- | --- | --- | --- | --- | --- |
| F1 | 110 | 400 | [400, 1200] | 1 | 4 |
| F2 | 210 | 400 | [200, 1000] | 2 | 3 |
| F3 | 160 | 260 | [800, 2000] | 3 | 1 |
| F4 | 60 | 350 | [500, 1300] | 4 | 2 |

The scenario of Figure 5 is divided into six independent time periods based on the flow activation and termination times. Based on this partitioning Table 2 shows the optimal bandwidth distribution using the proportional and maximizing utility definitions of fairness. Figure 6 illustrates bandwidth distribution among the flows using the maximizing utility definition of fairness. Per-flow resource distribution of Figure 6 was obtained through simulation and it very close to the optimal distribution of Table 2.

Table 2. Optimal per-flow resource allocation for simulation scenario of Figure 5

| Flow Name | Optimal flow rates for PROPORTIONAL fairness |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | [60, 110] | [110, 160] | [160, 210] | [210, 260] | [260, 330] | [330, 400] |
| F1 | 0 Kbps | 711.1 Kbps | 533.3 Kbps | 457.1 Kbps | 581.8 Kbps | 1066. 7 Kbps |
| F2 | 0 Kbps | 0 Kbps | 0 Kbps | 228.6 Kbps | 290.9 Kbps | 533. 3 Kbps |
| F3 | 0 Kbps | 0 Kbps | 1066.7 Kbps | 914.3 Kbps | 0 Kbps | 0 Kbps |
| F4 | 1400 Kbps | 888.9 Kbps | 1066.7 Kbps | 914.3 Kbps | 727.3 Kbps | 0 Kbps |
| Flow Name | Optimal flow rates for MAXIMIZING UTILITY fairness |  |  |  |  |  |
|  | [60, 110] | [110, 160] | [160, 210] | [210, 260] | [260, 330] | [330, 400] |
| F1 | 0 Kbps | 750 Kbps | 560 Kbps | 457.1 Kbps | 566.67 Kbps | 900 Kbps |
| F2 | 0 Kbps | 0 Kbps | 0 Kbps | 257.1 Kbps | 366.67 Kbps | 700 Kbps |
| F3 | 0 Kbps | 0 Kbps | 1040 Kbps | 885.7 Kbps | 0 Kbps | 0 Kbps |
| F4 | 1400 Kbps | 850 Kbps | 1040 Kbps | 885.7 Kbps | 666.67 Kbps | 0 Kbps |

Figure 6. Example of resource distribution using the X-BDS approach

To better understand the nature of the X-BDS scheme, let us carefully examine what happens during the time periods [210, 260] and [260, 330] seconds. At time 210 seconds, flow F2 activates causing congestion on links C2-C5 and C5-C3. As a result, core router C2 initiates the notification phase requesting flows F1, F2, and F3 to adjust their allocated rates because link C2-C5 is their bottleneck. Meanwhile, core C5 initiates notification phase and notifies Edge 1, Edge 2, and Edge 3 to throttle their flows. However, only Edge 3 adjusts transmission rate of its flow, because link C5-C3 is bottleneck only for flow F4. After the rate reduction, Edge 3 conducts the path probing, discovers that link C5-C3 is underutilized, and gradually increases transmission rate of F4 until all excess bandwidth is consumed.

At time 260 seconds, flow F3 terminates and link C5-C3 becomes a new bottleneck for flows F1, F2, and F4. Based on the path probing results, Edge 1 discovers that flows F1 and F2 send data at rates below their fair shares. Edge 1 increases allocated rates of F1 and F2 causing congestion on link C5-C3. Subsequently, F4 receives CN from C5 and reduces its sending rate.

## Evaluation of the Resource Management Mechanism

To evaluate the effectiveness of the resource management mechanism in achieving a fair distribution of the bandwidth, we define a metric called degree of fairness. The degree of fairness at time t for flow f is a measure of the ratio between the allocated rate Rft and the optimal allocated rate ROPTft:

DFft=1−1−RftROPTft,ifRftROPTft<20,otherwise		(14)

The degree of fairness is a statistic whose value lies between 0 and 1 and which shows how fairly the resources were distributed at a particular point in time. High degree of fairness values (e.g. 0.95 - 1.00) correspond to a situation when resources are distributed fairly and the allocated rates of the flows are close to the corresponding optimal rates. Small degree of fairness values correspond to a situation when resources are not distributed fairly and the allocated rates of the flows diverge from the corresponding optimal rates. We often average the degrees of fairness of all the active flows during a particular time period and refer to the obtained value as the average degree of fairness.

We compared the average degree of fairness achieved during the simulation for different probe period values. We varied the probe period between 1.0 and 2.0 seconds and averaged the degree of fairness values over 2 second intervals. Figure 7 presents two sets of graphs: one shows resource distribution through the whole simulation, the other zooms in on the time period [160, 330] seconds. The results presented in Figure 7 were collected using the maximizing definition of fairness. The proportional definition of fairness yields a similar outcome.

As Figure 7 shows, throughout the duration of the experiment, the X-BDS scheme achieves fair resource distribution: the values of the average degree of fairness vary between 0.95 and 1.00. Only in the events of flow activation or termination, the degree of fairness briefly drops below 0.95. More specifically, upon flow activation or termination, the average degree of fairness reaches lower values when the probe period value is large. For example, when flow F3 terminates at time 260 seconds the degree of fairness falls to 0.83 with the probe period of 2.0 seconds. However, when the probe period is 1.0 second the degree of fairness drops only to 0.91.

Figure 7. Achieved average degree of fairness

This happens because small values of the probe period cause the edge routers to probe the network more frequently and thus increase probability of discovering the path changes quickly. Meanwhile, the larger probe period values cause the edge routers not to discover the path changes for a longer period of time and thus cause the degree of fairness average to reach lower values. However, large values of the probe period cause the network feedback to provide a more accurate estimate of the excess bandwidth available on the path and thus results in less fluctuation of the flow rates. Our study suggests that the value of probe period between 1.2 and 1.4 seconds causes the resource distribution mechanism to perform reasonably well.

## Link Utilization

To compare the link utilization achieved by the maximizing utility and proportional fairness criteria, the simulation scenario of Figure 5 was slightly modified. The RBR of flow F4 was changed from [500, 1300] to [400, 450] Kbps. Figure 8 illustrates the achieved utilization on links C2-C5 and C5-C3 using both fairness criteria.

(a) Maximizing utility fairness

(b) Proportional fairness

Figure 8. Utilization of links C2-C5 and C5-C3

As Figure 8 shows, both fairness criteria are able to achieve close to 100 % link utilization. However, upon activation of flow F1 at time 110 seconds and termination of flow F2 at time 260 seconds, proportional fairness is unable to distribute all available bandwidth among active flows at once and relies on periodic path probing to distribute leftover bandwidth. It can be easily shown that proportional fairness fails to utilize all available bandwidth when the fair share of the flow is larger than its maximum requested rate [8, 10]:

Ckbfbk>Bf	(14)

Figure 9. Achieved utilization of link C5-C3 during the time period [258, 268] seconds

For example, at time 260 seconds the bandwidth of bottleneck link C5-C3 is distributed among flows F1, F2, and F4. Since the fair share of flow F4 is 1600*400/(400 + 200+ 400) = 640 Kbps, while its maximum requested rate is 450 Kbps, 190 Kbps of the bandwidth dedicated for F4 are left unused. Thus, using the proportional fairness criterion, the edge nodes often rely on periodic path probing to discover and utilize available bandwidth. On the other hand, the maximizing utility fairness does not suffer from this deficiency because by definition the fair share of the flow can be larger than the maximum requested rate only if maximum requested rate of all flows on the link is smaller than the link’s capacity.

bf+Ck−bkBf−bfBk−bk>Bf=>Ck>Bk	(15)

Figure 9 zooms in on the time period [258, 268] seconds to better illustrate this phenomenon. As Figure 9 shows, the maximizing utility fairness allows the edge routers to allocate all of the available bandwidth by time 262 seconds. On the other hand, the resource distribution using proportional fairness allocates only 90% of available bandwidth. The rest of the bandwidth is allotted within the span of the next 5 – 8 seconds based on the feedback from the periodic path probes. In summary, the simulation results showed that the proportional fairness criterion in certain cases fails to distribute available bandwidth completely. However, in such instances the overall performance of the X-BDS does not suffer much because the periodic path probing notifies the edge nodes about excess bandwidth available. As a result, the edge routers fairly distribute remaining resources within a short period of time.

Table 3. The timetable of CNG and CN message generation times

| CNG Generation Times |  |  | CN Arrival Times |  |  | CN Generation Times |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Edge 1 | Edge 2 | Edge 3 | Edge 1 | Edge 2 | Edge 3 | ##### Core 2 | Core 3 |
| -- | -- | 60.11 | -- | -- | -- | -- | -- |
| 110.16 | -- | -- | 110.23 | -- | 110.21 | 110.21 | 110.19 |
| -- | 160.11 | -- | 160.14 | 160.13 | -- | 160.12 | -- |
| 210.18 | -- | -- | 210.25 | 210.25 | 210.23 | 210.23 | 210.21 |
| -- | 260.63 | -- | 263.42 | -- | 263.40 | 263.40 | 263.39 |
| -- | -- | 330.61 | -- | -- | -- | -- | -- |

## Efficiency of Congestion Control

To evaluate the efficiency of the X-BDS congestion control mechanism we examine events that occur upon the flow activation/termination. Table 3 shows the timetable of the CNG and CN messages generation for scenario of Figure 5. As Table 3 shows, activation of flow F4 at time 60 seconds does not cause congestion in the network and as a result, the core routers do not generate CN messages. After flow F1 requests activation at time 110.16 seconds, core router C5 generates notification messages at time 110.19 seconds. Edge 3 receives CN message from C5 and adjusts allocated rate of flow F4 at time 110.21 seconds. At time 110.21 seconds, core router C2 receives CN message from C5 and forwards it to Edge 1. Finally, at time 110.23 seconds Edge 1 receives congestion notification and activates flow F1. Thus, flow F1 activates after flow F4 reduces its allocated rate and as a result, congestion on link C5-C3 is avoided.

A similar situation occurs upon activation of flow F2 at time 210 seconds. Edge 1 receives congestion notification and activates a new flow after edge routers Edge 2 and Edge 3 adjust allocated rates of their flows. As a result, congestion on links C2-C5 and C5-C3 is avoided. However, upon activation of flow F3 at time 160 seconds, Edge 2 receives congestion notification and activates flow F3 before Edge 1 has a chance to throttle flow F1. As a result, link C2-C5 could have become congested. However, since flow F3 was activated only 0.0028 seconds before flow F1 slowed down, the burst of excess traffic was too small to be noticeable. As a result, the core routers were unable to detect congestion.

Upon termination of flow F3 the situation changes. Edge 1 probes the network at time 261.81 seconds and receives a reply at time 261.99 seconds. Based on the path probing results, Edge 1 increases allocated rates of flows F1 and F2 and subsequently causes congestion on C5-C3. C5 observes congestion and generates notification messages only at time 263.39 seconds. As a result, Edge 3 receives the CN message and adjusts allocated rate of F4 only at time 263.40 seconds. In this case, congestion lasted for about 1.4 seconds and overall it took almost 2.8 seconds to re-distribute available resources fairly among active flows. The primary reasons for this delay are the time required by the core routers to discover congestion and length of the path probing period. Probing the network more frequently and using alternative means for identifying congestion may result in faster resource re-distribution in the event of flow termination.

Finally, upon termination of flow F4 at time 330 seconds, congestion does not occur because all the flows influenced by the change pass through the same edge router. Thus, when Edge 1 probes the network and discovers excess bandwidth, it distributes resources among its flows without causing congestion in the network.

(a). Overhead in bits

(b) Overhead in packets

Figure 10. Control Load Overhead

## Control Load Overhead

We define the control load overhead caused by the RDF protocol as the ratio between the amount of the control data and the total amount of data generated in the system. The overhead of the protocol was computed in terms of the number of packets and the number of bits generated in the system. Figure 10 illustrates how the overhead of the RDF protocol varies with the change of the path probing period.

As expected, the overall overhead of the X-BDS diminishes as the probe period decreases. For example, control traffic incurs only 0.03% of the bits overhead, when the edge routers probe the network every 2.0 seconds. In the worst case studied, when the edge routers generate probe messages every 1.0 seconds, the RDF protocol incurs only 0.06% of the bits overhead. However, the overhead in terms of packets is an order of magnitude larger than the overhead in terms of bits. The X-BDS packet overhead ranges from 1.7% to 3.5%. Such behavior is expected, since the size of a control packet is significantly smaller than the average size of a data packet.

- Discussion and Related Work

Most of the current architectures that support QoS in the Internet have emerged from various proposals by the Internet Engineering Task Force (IETF). In 1994, the IETF introduced Integrated Services [3] architecture, followed by the Differentiated Services [2] model in 1998. Although both of these approaches address the same problem of supporting quality of service in the Internet, they are different in terms of implementation and provided services. Integrated Services provides end-to-end guarantees on a per-flow basis, while DiffServ attempts to provide end-to-end guarantees based on per-hop assurances for a small set of predefined traffic classes. At the implementation level, Integrated Services requires per-flow management in the network core, while the Differentiated Services model employs a network architecture that pushes per-flow management to the network edges.

This paper describes a novel approach called Exact-RBR Bandwidth Distribution Scheme (X-BDS), which attempts to combine the advantages of the Integrated and Differentiated Services models by providing a framework for scalable support of per-flow bandwidth guarantees. The edge nodes in the X-BDS network manage per-flow information while the core routers deal only with the traffic aggregates. The X-BDS scheme aggregates flow requirements in the network core and distributes these aggregated requirements in the network, so that they may be used to obtaine information for fair sharing of available bandwidth among individual flows. The X-BDS employs a combination of periodic probing and explicit network feedback for distributing the aggregate flow requirements in the network.

The idea of using explicit network feedback for dynamic rate and congestion control is not new. In particular, the Explicit Congestion Notification (ECN) extension to IP [20] uses binary feedback to notify ECN-capable transports about congestion occurrences. Unlike the ECN extension, the network feedback in the X-BDS model not only notifies the edge routers about congestion but also carries additional information such as the arrival rate and aggregate RBR on the congested link. A similar idea is used in ATM networks for Available Bit Rate (ABR) congestion control [15], where the feedback carried by the resource management cells also includes rate information. However, the ABR congestion control relies on per-flow information stored in the network core and tries to achieve utilization goals first and only then seeks fairness. In contrast, the X-BDS model does not store per-flow information in the network core. Instead, the core routers maintain a soft state for the edge routers that send traffic though the outgoing links of the core router and an aggregate RBB for those links. Furthermore, the X-BDS approach tries to achieve utilization and fairness goals simultaneously: the edge nodes compute the fair shares of individual nodes so as to consume all bandwidth allocated for X-BDS traffic, and in the presence of excess bandwidth individual flows increase their transmission rates so as to preserve fairness. The Explicit Control Protocol (XCP) [14] generalized the ECN proposal by sending additional information about congestion. XCP also does not require per-flow information in the network core. However, unlike X-BDS, XCP is not a rate-based but a window-based protocol that separates utility control from the fairness control.

The X-BDS approach is designed primarily for support of per-flow bandwidth guarantees. A similar feedback-based idea of providing dynamic per-flow bandwidth allocation for elastic traffic sources called simple rate control algorithm was introduced in [13]. A traffic source is called elastic if it does not require a fixed rate and can adjust its transmission rate as needed. Unlike X-BDS, the boundary nodes in the simple rate control algorithm employ knowledge of the level of network congestion and the user utility functions to determine a fair resource distribution among elastic sources. The end users obtain the level of congestion through the explicit acknowledgements (ACK) that carry the number of congested links on a particular path.

The Stateless-Core approach [22] provides an interesting solution for supporting per-flow QoS without keeping per-flow information in the network core. The main idea of this scheme relies on the Dynamic Packet State (DPS), where control information is carried in the IP header of the data packets [23]. The routers use the DPS information to provide per-flow guarantees without maintaining per-flow state in the network core. The main differences between the X-BDS and Stateless-Core models are information storage in the network core, distribution of flow requirements, and resource allocation for individual flows. The Stateless-Core approach does not keep any additional information the network core, while the X-BDS requires each core router to maintain a “soft-state” for the edge routers that sends traffic though its links. However, in the DPS mechanism, used by the Stateless-Core approach, to carry flow requirements in the packet’s header requires additional processing of each data packet in the network core. On the contrary, the X-BDS employs a light-weight distributed message exchange protocol that carries control information separately from data and thus does not require additional per-packet processing. The RDF protocol shares aggregate flow requirements among the nodes in the network which requires negligible amount of processing in the network core and as shown in Section 9.5 introduces a small amount of traffic due to exchange of control information.

In the Stateless-Core approach, the core routers provide per-flow rate allocation via a FIFO queue with probabilistic drop, where the probability of dropping a packet is a function of the estimated rate carried in the packet’s header and the fair share at that router which is estimated based on measurements of the aggregate traffic [22]. In this scheme, if a flow transmits at a rate higher than its fair share rate, then non-conforming packets of the flow will be dropped in the network core. By traveling through the network, instead of being dropped right away at the boundaries, these non-conforming packets will waste valuable network resources such as buffer space, bandwidth, etc. The X-BDS model adjusts transmission rates of the flows before they enter the network, which avoids this problem. However, adjustment of per-flow rates in the core has the advantage of not being subject to the propagation delay of the notification messages required in the X-BDS model. Another disadvantage of the Stateless-Core model is its inability to distribute excess bandwidth due to use of the upper bound of the aggregate reservation for admission control, which could possibly lead to network underutilization [22]. On the contrary, the X-BDS approach fairly distributes excess bandwidth and maximizes network throughput. Both the stateless-Core and BDS approaches scale well to large networks.

Similarly to the simple rate control algorithm [13], the X-BDS approach can be used to support bandwidth guarantees and is most suitable for elastic sources that can tolerate and benefit from frequent changes of the allocated rates. FTP and video flows are examples of such elastic traffic. The main advantages of the X-BDS approach are scalability, ability to eliminate or even avoid congestion while maintaining high link utilization, and ability to fairly distribute available resources among individual flows, while supporting minimum bandwidth guarantees of each flow. In addition, the RDF protocol of the X-BDS approach is fairly simple and does not incur significant load overhead.

- Conclusions

This paper introduced a novel idea, called the Bandwidth Distribution Scheme, for support of scalable per-flow bandwidth guarantees. In the X-BDS approach, each flow that enters the network is guaranteed its minimum requested rate regardless of the network conditions. The leftover bandwidth is fairly distributed among the flows that can utilize it. The flows whose requests cannot be satisfied without violating guarantees of other active flows are not allowed to enter the network. The X-BDS approach relies on the network architecture where the edge routers maintain per-flow requirements while the network core deals with the traffic aggregates only. Such an architectural model enables the X-BDS to scale well to large networks. The Requested Bandwidth Range Distribution and Feedback protocol distributes aggregate flow requirements among the nodes in the network. The aggregate flow requirements provided by the RDF protocol support the admission control and resource management units of the edge routers. Admission control determines if a new flow can be admitted into the network, while the resource management unit computes the bandwidth fair shares for individual flows.

Evaluation of the X-BDS approach shows that the X-BDS is capable of supporting scalable and fair per-flow distribution of available bandwidth, which could be used for building services with per-flow bandwidth guarantees. Furthermore, the X-BDS supports high link utilization, allows the new flows to enter the network without causing congestion, eliminates congestion fast, and causes negligible overhead in the network. Even though the X-BDS is primarily suited for large elastic flows, in [9] we examined an optimization that allows the X-BDS to perform well in the presence of large number of small flows. Currently, we are investigating a number of additional mechanisms for improving the X-BDS approach. We also are examining the possibility of extending the X-BDS approach to a multi-domain environment, stability issues of the X-BDS approach, and the possibility of introducing the X-BDS approach into a mobile environment.

- References:

- A. Banchs, “User Fair Queuing: Fair Allocation of Bandwidth for Users,” In Proc. of IEEE INFOCOM’02, June 2002.

- S. Blake, D. Black, M. Carlson, E. Davies, Z. Wang, W. Weiss. "An Architecture for Differentiated Services," December 1998. IETF RFC 2475

- R. Braden, D. Clark, S. Shenker, “Integrated Services in the Internet Architecture: an Overview”, June 1994, IETF RFC 1633.

- L. Breslau, S. Jamin, S. Shenker, "Comments on the Performance of Measurement-Based Admission Control Algorithms," In Proc. of IEEE INFOCOM’00, March 2000.

- D. Clark, W. Fang, "Explicit Allocation of Best Effort Packet Delivery Service," IEEE/ACM Transactions on Networking, vol. 6, no. 4, pp. 362-373, August 1998

- W. Feng, D. Kandlur, D. Saha, and K. Shin "Understanding and Improving TCP Performance over Networks with Minimum Rate Guarantees," IEEE/ACM Transactions on Networking, Vol. 7, No. 2, pp. 173-187, April 1999.

- R. Gibbens and E. Kelly, “Measurement-based connection admission control”, In Proc. of 15th International Teletraffic Congress, Amsterdam, Netherlands, pp. 781-790, June 1997.

- V. Hnatyshin, "Dynamic Bandwidth Distribution Techniques For Scalable Per-Flow QoS," Ph.D. Thesis, University of Delaware, 2003.

- V. Hnatyshin and A.S. Sethi, "Reducing load distribution overhead with message aggregation," In Proc. of the 22nd IEEE International Performance, Computing, and Communications Conference, pp. 227-234, April 2003.

- V. Hnatyshin and A.S. Sethi, "Fair and Scalable Load Distribution in the Internet," In Proc. of the International Conference on Internet Computing, pp. 201-209, June 2002.

- V. Hnatyshin and A.S. Sethi, “Avoiding Congestion Through Dynamic Load Control,” In Proc. ITCom-2001, SPIE's International Symposium on The Convergence of Information Technologies and Communications, Denver, CO, pp. 309-323, Aug. 2001.

- V. Hnatyshin and A.S. Sethi,  “Estimation Based Load Distribution in the Internet,” under review.

- K. Kar, S. Sarkar, L. Tassiulas, “ A Simple Rate Control Algorithm for Maximizing Total User Utility,” In Proc. of INFOCOM’01, April 2001, pp. 133-141.

- D. Katabi, M. Handley, and C. Rohrs, “Internet congestion control for future high bandwidth-delay product environments,” In Proc. ACM SIGCOMM’02, Pittsburgh, PA, August 2002.

- S. Kalyanaraman, R. Jain, S. Fahmy, R. Goyal, and B. Vandalore, "The ERICA Switch Algorithm for ABR Traffic Management in ATM Networks," IEEE/ACM Transactions on Networking, Vol. 8, No. 1, February 2000, pp. 87-98.

- F. Kelly, P.B. Key, and S. Zachary, "Distributed Admission Control," IEEE Journal on Selected Areas in Communications, 18 (2000), pp.2617-2628.

- V. Kumar, T. Lakshman, D. Stiliadis,  "Beyond Best Effort: Router Architecture for the Differentiated Services of Tomorrow's Internet," IEEE Communications Magazine, pp. 152-164, May 1998.

- P. Marbach, “Priority Service and Max-Min Fairness,” In Proc. of IEEE INFOCOM’02, June 2002.

- OPNET Modeler. OPNET Technologies Inc.  http://www.mil3.com

- K. Ramakrishnan, S. Floyd, and D. Black, “The Addition of Explicit Congestion Notification (ECN) to IP”, September 2001, IETF RFC 3168.

- B. Radunovic and J. Le Boudec, "Unified Framework for Max-Min and Min-Max Fairness with Applications," Technical Report IC-200248, EPFL, July 2002.

- I. Stoica,  “Stateless Core: A Scalable Approach for Quality of Service in the Internet,” Ph.D. Thesis, Carnegie Mellon University, 2000.

- I. Stoica, H. Zhang, "Providing Guaranteed Services without Per-Flow Management," In Proc. ACM SIGCOMM, September 1999.

- H. Tzeng and K. Sui, "On Max-Min Fair Congestion Control for Multicast ABR Service in ATM," IEEE Journal on Selected Areas in Communications, Vol. 15, No. 3, pp. 545-556, April 1997.

- B. Vandalore, S. Fahmy, R. Jain, R. Goyal, and M. Goyal, “General weighted fairness and its support in explicit rate switch algorithms,” Computer Communications, Vol. 23, No. 2, pp 149-161, January 2000.

In this paper the terms link, interface, and interface to a link are often used interchangeably.