##### Engineering Electromagnetics 1			 Midterm Examination

#### Fall 2004

# September 29, 2004

12:40 – 2:40 PM

Student’s Name:

DIRECTIONS: Answer all questions. Report all work for credit. List all assumptions. This exam is OPEN BOOK & NOTES. Print your name on all pages. Remember to specify UNITS for all answers. Use proper NOTATION.

Part I:  (5 questions; 5 points each)

- Determine the wavelength at the following signal frequencies:

  - AM radio transmission at 1,060 kHz

  - FM radio transmission at 93.3 MHz

  - Airplane instrument landing system at 335 MHz

  - Satellite communications uplink at 6 GHz.

- Sketch the following fields:

  - F=zy

  - P=ρφ

- Write down any two of the Maxwell’s equations and provide a physical interpretation for them.

- How are the electric and magnetic fields as described by Maxwell’s equations related to each other under static conditions?

- Figures 5 (a) and (b) show static electromagnetic field distributions for various situations. Explain briefly whether each situation represents an acceptable electromagnetic field distribution.

Air

High-permeability

Magnetic Material

Magnetic

Field

Lines

Figure 5 (a)

Medium 1

Medium 2

y

x

Medium 1

Medium 2

y

x

Figure 5 (b)

Part II: (10 points)

- A box-shaped surface S0 enclosing a volume V0 contains a set of discrete (Q1, Q2 and Q3) and continuous (l, s and v) charge distributions as shown in Figure 2.

lO1, S01 and V01 are, respectively, portions of the line l1, surface S1 and volume V1 contained within S0. Describe a procedure for computing the electric field over the surface S0. Provide the relevant equation/s, but DO NOT solve it/them.

s

###### Figure 6

Part III: (15 points)

- Find an approximate value for the total charge enclosed in an incremental volume of 10-9 m3 located at the origin; D=e−xsinyx−e−ycosyy+2zz Cm-2.

- Find the amplitude of the displacement current density adjacent to a car antenna receiving an FM radio station where the magnetic field intensity is H=0.15cos3.123×108t−yzAm-1.