PAGE  
2

ECOMMS


 
Final Exam

          

   May 3, 2006
Spring 2006 






                    
 10:45 – 12:45
Student’s Name: 









DIRECTIONS: This exam consists of three parts – Part A, Part B and Part C. Answer all questions in all three parts. This exam is OPEN BOOK/NOTES. Calculators are permitted. Remember to specify UNITS for all answers. Use proper NOTATION. Show ALL WORK.

Part A (8 questions @ 5 points each = 40 points)

1. What is the difference between a time-limited signal and a band-limited signal? Can a communications signal be both time-limited and band-limited?
2. Describe the fundamental techniques by which digital signals are bandpass modulated.
3. Is the following signal a power signal or an energy signal?

[image: image1.wmf]î

í

ì

<

=

otherwise

t

t

w

0

3

3

)

(


Calculate its average normalized power and energy.
4. A single-digit 7-segment LCD display emits 0 with a probability of 0.25; a 1 and 2 with a probability of 0.15 each; 3, 4, 5, 6, 7, and 8 with a  probability of 0.07 each; and a 9 with a probability of 0.03. Find the average information for this source.
5. What are the limitations of a coherent detector in demodulating AM signals? How can these be overcome?
6. What are the advantages of a PLL over a slope-detector in demodulating FM-signals?
7. Suggest ideal summer-time activities for Mr. Ciocco.

8. Compute the state transition matrix of the following M-ary constellation.

[image: image2]
Part B

9. Draw a block diagram for generating the following band-limited signal:

[image: image3.wmf]ï

î

ï

í

ì

£

=

-

-

otherwise

f

f

f

M

0

kHz

 

2

)

10

(

 

sinc

10

)

(

3

3


For each block, provide equations for describing the input and output, and sketch the spectrum.

Extend the block diagram so that 
[image: image4.wmf])

(

f

M

is frequency-translated by 100 kHz.
Suppose 
[image: image5.wmf])

(

f

M

is sampled at a rate of 100 kHz. Sketch the spectrum of the sampled waveform; provide an equation describing it.

 (30 points)

Part C

10. Software Defined Radio
The Software Defined Radio (SDR) uses DSP modules, FPGAs and software to create modulated signals for transmission and to demodulate signals at the receiver. The advantage of this system is that new receivers can be configured just by downloading code – no reassembly of the system hardware is required. The block diagram of a typical SDR receiver is shown below.

[image: image6]
SDRs have found applications mainly in the military, where there is constant demand for reconfigurable communications. The bottleneck that prevents widespread use of this system in the commercial sector is the requirement for the Analog-to-Digital-Converter hardware to process signals modulated with GHz carrier frequencies – most currently available ADCs cannot process signals at this speed.
Propose solutions to this problem by frequency-downshifting the input RF signal so that it can be processed by the ADC/DSP modules. In particular, speculate how such frequency translation can occur by exploiting the effect of signal sampling (refer to your solution for the last question in Part B). Provide block diagrams, equations and a convincing argument in favor of your solution. Also discuss its limitations.
(30 points)




























I








Q





11





01





10





00






























































YOUR CODE HERE!








ADC/DSP





Receiver RF Front End











_1208146815.unknown

_1232041846.unknown

_1208146293.unknown

